# kiro_wiring – power rewiring design for the Kiro / SO-101 mobile dual-arm robot

Created 2026-10-22. All documents are in English.

| File | Content |
| --- | --- |
| Kiro_SO101_Power_System_Design_RevA.docx | Main deliverable. Complete electrical design package: context, load budget and calculations, SMPS and converter selection, protection and E-stop, five schematic sheets with reference designators and ratings, designator list, net list, cable schedule, assembly notes, parts list, image sources |
| context.docx | Project context: what the robot is, people and milestones, hardware inventory with serial numbers, current power and USB layout, electrical rules learned, goals, open questions |
| power_calculation.docx | Load budget per device and rail, SMPS and buck sizing, fuse groups, wire gauges and voltage drops, connectors, protections, E-stop, battery path, bring-up checklist |
| parts_list.docx | Bill of materials with specs, quantities, search terms and rough prices |
| diagrams/schematic_sheet1..5.png (+ .svg) | Electrical schematics: 1 AC input, SMPS, 12 V distribution, E-stop; 2 Jetson and 7.2 V follower rails; 3 5 V logic rail; 4 12 V wheel rail; 5 connector pinouts. Source: schematic.py (Python, schemdraw) |
| diagrams/block_diagram.png, power_diagram.png, power_diagram_tb.png, wiring_diagram.png | Graphviz overview diagrams (.dot sources alongside) |
| images/web/ | Reference photos used in the Word document, with sources.json listing origin and licence |
| markdown_source/ | Original Markdown versions of the text documents |

## Design in one paragraph

One Mean Well LRS-350-12 (12 V, 29 A) replaces all wall adapters. The wheel servos and the Jetson run directly from 12 V (the Jetson jack accepts 9–20 V). Two 8 A class buck converters make 7.2 V for the left and right follower arms (servo limit 8.0 V), one 5 A buck makes 5 V for the USB hub, the lidar and both leader arms. Two blade fuse groups and a GND bus bar distribute power in a star: group A (Jetson, 5 V buck, spare) is always on, group B (wheels, both follower bucks) hangs behind a 40 A relay driven by a mushroom E-stop and a key switch, so a stop kills motion while the Jetson, hub, lidar and leaders stay powered. Design load is 17 A continuous and 27 A worst-case peak on the 12 V bus.

## Re-generating the drawings

cd diagrams && for f in block_diagram power_diagram power_diagram_tb wiring_diagram; do dot -Tpng -Gdpi=130 $f.dot -o $f.png; dot -Tsvg $f.dot -o $f.svg; done

python3 schematic.py        # needs: pip install schemdraw matplotlib