# Kiro Robot Project – Context

Written 2026-10-22 from the project records on this Jetson (~/so101 repository, its CLAUDE.md, docs/, the company handover package of 2026-10-15, and the Claude Code session history). Purpose of this folder: plan the rewiring of the robot's power system (one SMPS + buck converters instead of a pile of wall adapters) and give the company/students a clear picture of the machine.

Naming note: in the records "Kiro" (키로) is the company-side group for which the robot and the student curriculum are being prepared. The robot itself is called SO-101 mobile dual-arm robot in the repository and the handover documents. Both names refer to the same project.

## 1. What the project is

An XLeRobot-style mobile manipulator built around one NVIDIA Jetson Orin Nano 8 GB:

- two SO-101 leader arms (moved by hand) and two SO-101 follower arms (copy the leaders),

- a mecanum-wheel base driven by four Feetech ST3215-C018 bus servos,

- an Orbbec Gemini 335 RGB-D camera (top view) and a Suyin USB webcam on the right wrist,

- a Slamtec RPLIDAR C1 2D lidar (tested, not yet mounted),

- a USB gamepad for driving the base.

Use cases: leader-follower teleoperation, recording imitation-learning datasets (LeRobot 0.6.1, ACT policy), driving the base (ROS 2 Jazzy), and later autonomous navigation (SLAM / Nav2). The robot is also the platform for a student curriculum (교안) requested by the company; the students are meant to modify the data-collection and training parts of the code.

### People and milestones

| Date | Event |
| --- | --- |
| 2026-09-07 | Dual-arm teleoperation working (one process per arm), udev fixed names, GUI control panel |
| 2026-09-15 | AMR extension reviewed: mecanum + Gemini 335 + RPLIDAR C1 + ST3215-C018, LeRobot upgraded to 0.6.1 |
| 2026-09-16 | Wheel servos ID 7–10 assigned, wheel board registered as /dev/so101_base, 12 V 6 A bench adapter |
| 2026-09-19 | Base mounted on chassis, 2-chain wiring, floor driving verified (forward, strafe, rotate), gamepad teleop |
| 2026-10-12 | Progress report to the company, USB hub replaced by VIA VL815, arms confirmed running on 5 V |
| 2026-10-15 | Technical handover package SO101_기술자료_2026-10-15.zip delivered (system, code, training, interfaces, Q&A) |
| 2026-10-21 | Live demo for the company at 16:00, zero-base install of a spare Jetson, hardware test wizard, right-arm data collection prepared |
| 2026-10-22 | This folder: power rewiring plan (SMPS + buck converters) |

Owner/developer: 이상원 (Donghwa University, student researcher). Company side: Kiro (client group) and Indi Solution (curriculum coordinator). Standing project rules from the records: keep CLAUDE.md updated, commit helpful changes, never push (no remote), the user runs sudo steps themselves.

## 2. Hardware inventory (as of 2026-10-21)

| Item | Model | Qty | Key facts | Identification |
| --- | --- | --- | --- | --- |
| Main computer | NVIDIA Jetson Orin Nano Super Developer Kit, 8 GB | 1 | JetPack 7.2.1, Ubuntu 24.04, CUDA 13.2, power mode MAXN SUPER, Samsung 990 1 TB NVMe boot | bundled 19 V barrel adapter |
| Spare computer | Jetson Orin Nano dev kit (unopened) + 1 TB NVMe | 1 | zero-base install done 2026-10-21 via Jetson ISO | – |
| Follower arms | SO-101 Follower | 2 | 6 × STS3215 each (ID 1–6). 7.4 V type: Max_Voltage_Limit 8.0 V (register read). Run on 5 V so far (5.2–5.3 V measured) | /dev/so101_left_follower S/N 5B3D048978, /dev/so101_right_follower S/N 5B14111917 |
| Leader arms | SO-101 Leader | 2 | 6 × STS3215 each, torque off, position read only. Mixed limits (ID 2 = 8.0 V, others 12.0 V) | /dev/so101_left_leader S/N 5B14029523, /dev/so101_right_leader S/N 5B14113246 |
| Wheel servos | Feetech ST3215-C018 (12 V, 30 kg·cm, 1:345) | 4 | input 7.4–12.6 V nominal, Max_Voltage_Limit 14.0 V, stall 2.7 A each, velocity mode | IDs FR 7, FL 8, RR 9, RL 10 on /dev/so101_base S/N 5B3E091473 |
| Servo bus adapters | CH343 USB-UART boards (USB 1a86:55d3, "USB Single Serial") – most likely Waveshare Bus Servo Adapter (A): 2 servo ports, DC jack 5.5×2.1 mm, screw power terminal | 5 | one adapter = one TTL half-duplex bus at 1 Mbps. Logic is USB powered; the power terminal only feeds the servo VCC pins | serial-number based udev links above |
| RGB-D camera | Orbbec Gemini 335 | 1 | USB 3.2 Gen1, bus powered, bMaxPower 896 mA, peak < 6.5 W, colour 1280×720 + depth 848×480 @ 30 fps | S/N CP0F46300035, fw 1.4.60, body port 1 |
| Wrist camera | Suyin HD webcam (1e45:8022) | 1 (+1 planned left) | needs 500 mA; unpowered hubs reject it, body port works | body port 2 |
| 2D lidar | Slamtec RPLIDAR C1 + CP2102N adapter | 1 | 5 V, 230 mA / 1.15 W typical, UART 460800, /scan 10 Hz 720 pts | /dev/rplidar, on the hub |
| USB hub | VIA VL815 USB 3.0, self-powered (own adapter) | 1 | replaced a Genesys 05e3:0608 hub that failed to enumerate the serial boards | body port 3 |
| Gamepad | cheap USB pad in Xbox 360 XInput mode (045e:028e) | 1 | /dev/input/js0; currently on the hub (worked after the VL815 swap) | – |
| Power (today) | 19 V Jetson adapter, 5 V adapter(s) for the arm boards, 12 V 6 A adapter for the wheel board, hub adapter | – | see section 3 | – |

Servo joint order on every arm bus: 1 shoulder_pan, 2 shoulder_lift, 3 elbow_flex, 4 wrist_flex, 5 wrist_roll, 6 gripper. All servos are model 777 (STS3215 family), firmware 3.10, 4096 steps per turn, protocol 0 (little-endian).

## 3. Current power architecture (as-is, before rewiring)

Power is split into three buses plus the hub, each with its own wall adapter:

| Bus | Current source | Load and measured values | Rules learned |
| --- | --- | --- | --- |
| Arm servos (4 arms, 24 servos) | 5 V adapter(s) into the arm boards' DC jacks | teleop works at 5 V, Present_Voltage 5.2–5.3 V | Followers must never see 12 V (limit 8.0 V). A 7.4 V supply gives more torque but was not required |
| Wheel servos (4) | 12 V 6 A adapter (bench use only) | no-load 46–176 mA per wheel, floor driving a few hundred mA per wheel, servo voltage 12.2–12.4 V, 27–30 °C | 4-wheel stall ≈ 10.8 A exceeds 6 A; a 12 V 10 A class supply or battery was already planned |
| Jetson | bundled 19 V adapter | MAXN SUPER mode | if the Jetson reboots when the camera is plugged, suspect the adapter first |
| USB hub | own adapter (voltage not recorded – check the label) | powers 5 serial boards' logic, lidar, gamepad, dongle | power the hub before plugging devices |

Other electrical facts already established:

- All grounds are already common because every servo board and sensor is connected to the Jetson by USB.

- The servo connector is Molex 5264 (2.5 mm pitch) 3-pin, pin order GND / VCC / SIGNAL, contacts rated about 3 A with 22 AWG. Four wheel servos in one daisy chain would overload the first connector, so the base is wired as two chains: board port 1 → ID 7 (FR) → ID 9 (RR), board port 2 → ID 8 (FL) → ID 10 (RL).

- The adapter board's barrel jack is a 5 A class part; 10 A must go through the screw terminal, not the jack.

- A paper that ran XLeRobot on the same Orin Nano reported a brown-out (12.2 V collapsing to 0.3 V) when wheels, arms and Jetson shared one line, and fixed it by separating the buses.

- The lidar is 5 V only: never near 12 V wiring, sensor cable connected only with USB unplugged.

- Gemini 335 goes straight into a Jetson USB 3 port with the bundled cable; never through a hub or a charging cable.

- Software voltage guards: arm_tuck.py and the mode tester treat a servo voltage below 4.5 V as "power off" and refuse to move.

- No hardware emergency stop exists yet (listed as "planned" in the handover package).

### USB layout (2026-10-21)

| Jetson body port | Device |
| --- | --- |
| 1 | Orbbec Gemini 335 (USB 3, 5 Gbps) – top view camera |
| 2 | Suyin wrist webcam (right arm) |
| 3 | VIA VL815 hub → 5 × CH343 servo boards, RPLIDAR C1 adapter, gamepad, keyboard dongle |
| 4 | free |

The four body ports share one on-board Realtek hub (USB 3 and USB 2 bandwidth shared).

## 4. Software that touches power or safety

- LeRobot 0.6.1 (venv ~/lerobot-venv): lerobot-teleoperate 60 Hz, lerobot-record 30 Hz; followers torque on, position mode, P16/I0/D32; gripper torque limited to 50 %. Torque is released on disconnect, so followers fall under gravity when stopped.

- ROS 2 Jazzy (~/ros2_ws, package so101_base): mecanum_base_node at 20 Hz; velocity limits 0.3 m/s and 1.5 rad/s; watchdog sends zero velocity 0.5 s after the last /cmd_vel; torque off on SIGINT; refuses to start if any wheel servo does not answer a ping.

- GUIs: control panel, servo tester (shows voltage, current, temperature live), mode tester (mode A record / mode B drive with pre-checks), hardware test wizard, camera test, record GUI.

- udev rules bind every serial board to its USB serial number, so USB ports can be moved freely.

## 5. Goal of the rewiring

- Replace the separate wall adapters with one mains SMPS and derive the other rails with buck converters.

- Give every rail a fuse, proper wire gauge and connectors, and a hardware E-stop for the motion rails.

- Supply the followers at their proper 7.4 V class voltage (set to 7.2 V) instead of 5 V, giving more torque while staying below the 8.0 V servo limit.

- Keep the Jetson, the hub, the lidar and the leaders on a rail that is not cut by the E-stop, so the computer survives a stop.

- Keep the design battery-ready: a 12 V main bus lets a regulated 12 V power station (XLeRobot uses an Anker SOLIX C300) replace the SMPS later without any other change.

The result of this task is in the companion files (all in this folder):

- Kiro_SO101_Power_System_Design_RevA.docx – the complete design package: context, calculations, schematics (5 sheets), designator and net lists, cable schedule, parts list, image sources

- context.docx, power_calculation.docx, parts_list.docx, README.docx – the individual documents as Word files

- diagrams/schematic_sheet1..5.png|svg – electrical schematics (sheet 1 AC input and distribution, 2 Jetson and follower rails, 3 logic rail, 4 wheel rail, 5 connector pinouts); source schematic.py (schemdraw)

- diagrams/block_diagram.png, power_diagram.png, power_diagram_tb.png, wiring_diagram.png – Graphviz overview diagrams with .dot sources

- images/web/ – downloaded reference photos with sources.json (origin and licence of every image)

- markdown_source/ – the original Markdown text of the documents

## 6. Things to confirm on the bench before building

- The exact model printed on the five servo adapter boards (assumed Waveshare Bus Servo Adapter (A)).

- The hub adapter's voltage and current (5 V or 12 V input?), to decide which rail feeds the hub.

- How many 5 V adapters are in use today for the arm boards, and their connector types (to reuse plugs).

- The bundled Jetson adapter rating (expected 19 V, about 45 W) for comparison with the 12 V feed.

- Cable lengths from the planned power box to each arm board, the wheel board and the Jetson.

- Robot mass with arms, to refine the wheel current estimate under load.

- Whether the left wrist camera will be added (one more 500 mA USB load on a body port).

## 7. Source documents (on the Jetson)

- ~/so101/CLAUDE.md – living project log (hardware table, AMR section, power notes, USB hub history)

- ~/so101/docs/handover/01_시스템_구성_및_운영_프로토콜.md – system configuration, power structure (2.3), USB (2.4), servo IDs and wiring (2.5), start/stop procedures (6.x)

- ~/so101/docs/handover/04_통신_인터페이스_및_포팅_규격.md – USB and servo bus protocol specifications

- ~/so101/docs/handover/05_기술_확인_QnA.md – answers to the company (camera path, "no STM32 board")

- ~/so101/docs/mecanum_base_assembly_refs.md – mecanum coupling references, connector ratings, 12 V supply and cable shopping list (2026-09-16)

- ~/so101/docs/wheel_servo_id_guide.md, docs/lidar_c1_test_guide.md, docs/gemini335_connection_guide.md – device-level power and safety notes

- ~/so101/docs/hw_test_guide_2026-10-21.md, docs/demo_runbook_2026-10-21.md – test and demo procedures

- Handover package on the Desktop: SO101_기술자료_2026-10-15.zip